Describes data transmission between nodes

Most common: bus, ring

Bus logical topology

Signals travel from one device to all other devices

May or may not travel through intervening connectivity device

Bus logical topology used by networks with:

Physical bus topology & Star, star-wired bus topology

Ethernet

Ring logical topology

Signals follow circular path

Ring logical topology used by networks with:

Pure ring topology & Star-wired ring hybrid physical topology

Token ring

Bus topology

Bus -Single cable Connecting all network nodes No intervening connectivity devices - One shared communication channel

Physical medium  -  Coaxial cable

Passive topology  -  Node listens for, accepts data  -  Use broadcast to send

Bus topology

Broadcast domain -Node communicates using broadcast transmission

Terminators - 50-ohm resistors - Stops signal at end of wire

Signal bounce - Signal travel endlessly between two network ends

One end grounded - Removes static electricity

Advantages

Relatively inexpensive

Disadvantage

Does not scale well

Difficult to troubleshoot

Not very fault tolerant

Ring topology

Node connects to nearest two nodes

Circular network

Clockwise data transmission

One direction (unidirectional) around ring

Active topology

Workstation participates in data delivery

Data stops at destination

Physical medium

Twisted pair or fiber-optic cabling

Star topology

Node connects through central device

Physical medium

Twisted pair or fiber-optic cabling

Single cable connects two devices

Require more cabling, configuration

Advantage

Fault tolerance

Centralized connection point affects LAN segment

Scalable

Star-wired bus topology

Workstation groups

Star-connected devices

Networked via single bus

Advantage

Cover longer distances

Easily interconnect, isolate different segments

Drawback

Cabling, connectivity device expense

Basis for modern Ethernet networks

Figure 5-5 A star-wired bus topology network

- topology describes how the parts of a whole work together

- most network diagrams provide broad snapshots of a network’s physical or logical topology

- physical topology or network typology, mostly applies to hardware and describes how computers, other devises, and cables fit together to form the physical network

  - the physical topology used by the network is called a star topology because all devise connect to one central device, the switch

  - the physical layout of the media, nodes, and devise on a network

  - a physical topology does not specify device types, connectivity methods, or addressing schemes

  - physical topologies are categorized into three fundamental shapes: bus, ring, and star

    - these shapes can be mixed to create hybrid topologies

- logical topology has to do with software and describes how access to the network is controlled, including how users and programs initially gain access to the network and how specific resources, such as applications and databases, are shared on the network

  - a characteristic of network transmission that reflects the way in which data is transmitted between nodes

  - a network’s logical topology may differ from its physical topology